KEYBOARD INTERFACE

------------------

- SINCLAIR JOYSTICKu emulation on cursor keys+CTRL(ALT+F12=ON / SHIFT+F12=OFF)

- programmable keys (F1-F9,CURSORS,CTRL)

- RESET (CTRL+ALT+DEL)

- NMI (ALT+PRINT SCREEN)

2. KEYBOARD INTERFACE – KEYS FUNCTION

-------------------------------------

PC keyboard:      ZX keys:

ESC               CS+SPACE                       Reset keyboard matrix

F1                CS+1         EDIT              Prog.0

F2                CS+2         CAPS              Prog.1

F3                CS+3         T.VIDEO           Prog.2

F4                CS+4         I.VIDEO           Prog.3

F5                EXT,SS+9     CAT               Prog.4

F6                EXT,SS+6     MOVE              Prog.5

F7                EXT,SS+7     ERASE             Prog.6

F8                EXT,SS+0     FORMAT            Prog.7

F9                none                           Prog.8

CURSOR LEFT       CS+5                           Prog.9

CURSOR RIGHT      CS+8                           Prog.10

CURSOR UP         CS+7                           Prog.11

CURSOR DOWN       CS+6                           Prog.12

CTRL              SS           SYMBOL_SHIFT      Prog.13

INS               SS+W         <>
DEL               CS+9         GRAPH

HOME              SS+Q         <=

END               SS+E         =>

PGUP              CS+3         T.VIDEO

PGDW              CS+4         I.VIDEO

TAB               SS+CS        EXTEND_MODE

CAPS_LOCK         CS+2         CAPS_LOCK

SHIFT             CS           CAPS_SHIFT

ALT               SS           SYMBOL_SHIFT
BACKSPACE         CS+0         DELETE

PRINT SCREEN      EXT,SS+K     SCREEN$
WIN95-LEFT        CS+ENTER

WIN95-RIGHT       SS+ENTER

WIN95-MENU        SS+SPACE

Keys F1-F9, cursors and CTRL are signed as „Prog.X“. Prog. Signed programmable keys. Number X is 0 - 13 (14 keys).

Joytick mod OFF: (default after power-on, or after SHIFT+F12)

----------------

PC keyboard:      ZX keys:

CTRL              SS           SYMBOL_SHIFT

CURR_UP           CS+7         UP

CURR_DW           CS+6         DOWN

CURR_LF           CS+5         LEFT

CURR_RG           CS+8         RIGHT

Joystick mod ON: (after ALT+F12)

----------------
PC keyboard:      ZX keys:

CTRL              0            FIRE

CURR_UP           9            UP

CURR_DW           8            DOWN

CURR_LF           6            LEFT

CURR_RG           7            RIGHT

Other keys:

-----------

NUM Lock          -     work as numeric ZX keys (0-9,enter,+,-,,,/,*

CTRL+ALT+DEL      -     RESET ZX Spectrum + reset keyboard matrix

CTRL+ALT+ESC      -     RESET keyboard matrix only

ALT+PRINT_Screen  -     NMI

SHIFT+Fl0   -  keys layout on PC keys as on real AT keyboard

ALT+F10     -  keys layout on PC keys as on real ZX keyboard

ALT+F12     -  switch cursor keys to SINCLAIR JOYSTICK mode(6-9)and CTRL+0

SHIFT+F12   -  switch cursor keys to ZX cursor mode (5-8 with shift)and CTRL+SS